# ECA56 Advanced Wireless Communication Systems

CO1 Integrated Engineering Analysis and Planning Assignment – Solved Report Evolution, cellular planning, spectrum use and interference management

| Assessment format | Duration | Maximum marks | Bloom levels |
| --- | --- | --- | --- |
| Open-ended engineering design report | 1 week | 100 | K4 Analyze · K5 Evaluate · K6 Create |

Each stage question is shown first, with its answer below it, mapped to the rubric. Flow: requirements → analysis → calculations → comparison → recommendation.

| Assessment criterion (rubric) | Marks | Answered in |
| --- | --- | --- |
| Generational evolution and interpretation of CO1 concepts | 15 | Stage 1 |
| Cell geometry, reuse and capacity-expansion analysis | 15 | Stage 2 |
| Traffic and channel-capacity calculation | 15 | Stage 3 |
| Mobility and handoff analysis | 15 | Stage 4 |
| Interference calculations and mitigation reasoning | 20 | Stage 5 (Part A) |
| Spectrum-band comparison, ITU coordination and integrated recommendation | 20 | Stage 5 (Part B) and Stage 6 |
| TOTAL | 100 |  |

## 0. Requirements and global assumptions

Problem. A regional operator must plan the next stage of its network for a planning area of 1,000 subscribers (0.08 Erlang each in the busy hour) using hexagonal cells of radius 2 km, reuse cluster N = 7, a blocking target of ≤ 2 %, a 60 km/h mobility case, a minimum SINR of 15 dB, and a choice among 900 MHz, 3.5 GHz and 28 GHz spectrum.

Global assumptions (more are stated per stage): free-space propagation only where the task specifies FSPL; c = 3 × 108 m/s; thermal noise is not given and is therefore excluded from SINR (so “SINR” here equals the signal-to-interference-plus-adjacent ratio, an upper bound on the true SINR); all dBm/dB values are treated as single representative snapshots; textbook values quoted from general references are labelled as typical.

Stage 1 – Explain the evolution of mobile systems

QUESTION

Trace the progression from 1G through the 6G vision using the examples in the planning data. For each generation, identify its main service capability, access or radio technology where specified, and one important limitation or engineering change that motivated later generations. Explain how this progression changed the role of spectrum, capacity and interference management. Calculate the wavelength at 800 MHz and the idealized number of 30 kHz channels in 10 MHz. State the practical qualification to that idealized result. Relate the 2G move to digital transmission and TDMA to an improvement over 1G analog service. Deliverable: a concise generational comparison table and a short synthesis paragraph.

ANSWER

### Generational comparison table

| Gen. | Main service capability | Access / radio technology (from data) | Key limitation → change it motivated |
| --- | --- | --- | --- |
| 1G AMPS | Analog voice only | FDMA, 30 kHz analog channels, 800 MHz band, 10 MHz total | Low capacity, poor spectral efficiency, little security (eavesdropping, cloning), incompatible systems → digital transmission |
| 2G GSM | Digital voice, SMS, low-rate circuit data | TDMA/FDMA: 200 kHz carrier, 8 slots, 900 MHz | Circuit-switched, very low data rate → packet data and wider carriers |
| 3G UMTS | Voice plus mobile data / early mobile broadband | 5 MHz carrier (wideband CDMA) | Interference-limited CDMA, data rates and latency still modest → OFDMA, MIMO, all-IP |
| 4G LTE | All-IP mobile broadband (video, apps) | OFDMA and MIMO | Sub-6 GHz spectrum scarce; latency/reliability not enough for critical services → service-differentiated, wider-band 5G |
| 5G NR | eMBB, URLLC and massive IoT | Mid-band and mmWave, beamforming | mmWave blockage and short range; dense-site cost and energy; sensing/AI not native → 6G vision |
| 6G (vision) | THz links, AI-native networking, holographic communication, tactile internet, ISAC (joint sensing and communication) | Not yet standardised – research vision | Extreme path loss and hardware at THz, energy use, standardisation and spectrum still to be settled |

### Calculations

Wavelength at 800 MHz: λ = c / f = (3 × 108 m/s) / (800 × 106 Hz) = 0.375 m (37.5 cm).

Idealized channel count: Nch = Btotal / Bch = 10 MHz / 30 kHz = 333.3 → 333 channels (whole channels only).

Practical qualification. This figure ignores guard bands and duplexing. It treats the 10 MHz as one block of simplex channels. A real AMPS-type system is duplex: each call needs a paired uplink and downlink channel, so 10 MHz total (5 MHz each way) supports only about 166 duplex channels (5 MHz / 30 kHz = 166.7). Guard bands, separate control channels and the reuse factor reduce the number of channels usable in any one cell much further – with N = 7, only about 166/7 ≈ 23 voice channels per cell in this idealized case. Hence 333 is an upper bound, not a design value.

2G versus 1G. By carrying speech as a digital bit stream, GSM could use speech coding (fewer bits per call), channel coding and equalisation to tolerate a lower carrier-to-interference ratio than analog FM (typically about 9 dB for GSM versus about 18 dB for AMPS, quoted as typical textbook values). A lower tolerated C/I allows a smaller reuse cluster, so the same spectrum is reused more often. TDMA places 8 users on each 200 kHz carrier, i.e. about 25 kHz of carrier bandwidth per slot (200/8), comparable to AMPS’s 30 kHz per call, but with time-division sharing, digital error control and encryption. In the 10 MHz example this gives 10 MHz / 200 kHz = 50 carriers × 8 slots = 400 slots (simplex, ideal, before guards and control overhead), against 333 analog channels. 2G also brought authentication/encryption, SMS, and roaming standards.

Synthesis. Across the generations the role of spectrum moved from a scarce resource carved into narrow, exclusively assigned channels (1G FDMA) to one that is shared in time (2G TDMA), code (3G CDMA) and frequency–space (4G OFDMA, MIMO), and then to a portfolio of low-, mid- and mmWave bands with beamforming (5G) and a THz and sensing-enabled vision (6G). Capacity growth came from three levers: more spectrum (wider carriers from 30 kHz to 5 MHz to hundreds of MHz), higher spectral efficiency (digital coding, OFDMA, MIMO) and more spatial reuse (smaller cells, sectors, beams). Interference management changed accordingly: in 1G it was handled almost entirely by fixed frequency planning (reuse N = 7); in 2G by tighter reuse plus power control and frequency hopping; in 3G by power control and soft handoff because every user shares the same frequency; in 4G/5G by scheduling, coordination and beam steering. Each step therefore made the system more dependent on dynamic, software-driven interference control and on dense, heterogeneous deployments.

## 2) Stage 2 – Analyze cell coverage and frequency reuse

QUESTION

Using the hexagonal planning model, calculate the area of one cell and the total area of the 19-cell cluster. For N = 7, calculate the frequency reuse fraction and the reuse distance D = r√(3N). Explain the assumptions behind the hexagonal approximation and distinguish geometric coverage from guaranteed radio coverage. Explain how cell splitting, sectoring, microcells, picocells and femtocells can change coverage and capacity. Recommend a suitable capacity-expansion measure for the regional operator and identify its effects on interference and handoff frequency.

ANSWER

### Calculations

Area of a regular hexagon of circumradius r: Acell = (3√3 / 2) r² = 2.598 × (2 km)² = 10.39 km². Total area of the 19-cell cluster: A19 = 19 × 10.39 = 197.45 km².

Frequency reuse fraction: 1/N = 1/7 = 0.143 (each cell may use about 14.3 % of the total channel set).

Reuse distance: D = r√(3N) = 2 km × √21 = 2 × 4.583 = 9.17 km (reuse ratio D/r = q = 4.58).

Interpretation. The nearest co-channel cell centres are 9.17 km apart, i.e. 4.58 cell radii. A larger D/r reduces co-channel interference but fewer channels are available per cell; N = 7 is the classic compromise between the two.

### Hexagon assumptions and geometric versus radio coverage

The hexagon is a convenient tile: it is the regular polygon closest to a circle that tiles a plane with no gaps or overlaps. It assumes flat homogeneous terrain, omnidirectional antennas of equal height and power at each site, identical propagation law in all directions, uniform user density and sites placed on a regular grid. Geometric coverage is the idealized area of the hexagon. Guaranteed radio coverage is a statistical statement – the area where the received level exceeds a threshold with a given probability (for example 90 % at the cell edge) after allowing for shadowing, fast fading, clutter, building penetration and terrain. A link budget and measurements are required.

### Capacity-expansion techniques

| Technique | Effect on coverage and capacity | Interference / handoff impact |
| --- | --- | --- |
| Cell splitting | Radius halved → cell area ¼ → about 4× as many cells in the same area → about 4× capacity. Transmit power is lowered (about 12 dB for path-loss exponent 4) | D/r unchanged, so C/I is preserved; handoff rate rises roughly in proportion to 1/r |
| Sectoring (e.g. 120°) | Same site, channels divided among sectors; keeps coverage while allowing a smaller N or more channels reused | First-tier co-channel interferers fall from 6 to about 2 for 3 sectors → C/I improves; adds sector-to-sector handoffs and loses some trunking efficiency |
| Microcells | Radius tens to few hundred metres; street-level hotspots and corridors | Isolated by building shadowing but needs careful planning; frequent handoffs for fast users (umbrella macro cell needed) |
| Picocells | Indoor/enterprise, venue-scale cells; adds capacity inside buildings | Low leakage if walls isolate; handoffs mainly for walking users |
| Femtocells | Home/office user-deployed access points; offload macro and fix indoor coverage | Uncoordinated deployment can create interference and handoff/security issues; need self-organising control |

Recommendation. The Stage 3 result shows that the 1,000-subscriber planning area needs only 92 channels, i.e. one 100-channel base station. Capacity is therefore not the immediate bottleneck; the Stage

5 interference result (SINR 13.8 dB, below the 15 dB target) is. The recommended first measure is three-sector (120°) sectorisation of the macro sites, with antenna down-tilt: it reuses existing sites, cuts co-channel interference, and provides capacity headroom at low cost. As demand grows beyond one

site’s capacity, selective  cell  splitting  and  microcell/small-cell  overlays  at  traffic  hotspots become the growth path (using 3.5 GHz and 28 GHz; see Stage 6). Effects: interference decreases (about 2 rather than 6 first-tier co-channel interferers); handoff frequency increases because sector boundaries add intra-site handoffs and, for splitting/microcells, boundary crossings per minute scale roughly as 1/r. The handoff load must therefore be checked (Stage 4).

3) Stage 3 – Estimate traffic capacity

QUESTION

Calculate the offered busy-hour traffic for the planning area. Find the Erlang B blocking probability with 100 channels, then determine the smallest channel count that meets the 2 % target. Estimate how many 100-channel base-station channel groups are needed to serve that channel requirement, and state why this is only a first-order estimate rather than a complete site plan. Explain the capacity, blocking and infrastructure trade-offs in your result. State at least one assumption behind the offered-traffic model.

ANSWER

### Model and inputs

Offered traffic: A = Nsub × a = 1,000 × 0.08 Erlang = 80 Erlangs (≈ 80 simultaneous calls on average in the busy hour).

Erlang B (blocked calls cleared) with A = 80 E and C channels, computed with the numerically stable recursion

B(A,0) = 1, B(A,k) = A·B(A,k−1) / [k + A·B(A,k−1)], k = 1…C

k=0..C

(equivalent to B = (AC/C!) / Σ    Ak/k!). Spreadsheet or script evaluation gives:

| Channels C | Blocking B(80, C) | Meets ≤ 2 %? | Carried traffic A(1−B) | Channel utilisation |
| --- | --- | --- | --- | --- |
| 80 | 8.41 % | No | 73.3 E | 91.6 % |
| 90 | 2.62 % | No | 77.9 E | 86.6 % |
| 91 | 2.25 % | No | 78.2 E | 85.9 % |
| 92 | 1.92 % | Yes (smallest) | 78.46 E | 85.3 % |
| 100 | 0.40 % | Yes | 79.68 E | 79.7 % |

Figure 1 – Erlang B blocking versus channels for 80 Erlangs (script output; method as above).

Results. (i) With C = 100: B ≈ 0.40 %, well inside the target. (ii) Smallest channel count meeting B ≤ 2

%: C = 92 (B = 1.92 %); C = 91 gives 2.25 %, which fails. (iii) Channel groups: 92 / 100 → ⌈0.92⌉ = 1 base-station group of 100 channels, leaving 8 spare channels (8 %).

### Why this is only a first-order estimate

It treats the whole area as one pooled group of channels. A real site plan must also provide: frequency reuse (each cell gets only about 1/N of the spectrum), the spatial distribution of users and hotspots, sectorisation (channels split between sectors reduces trunking efficiency), coverage and link-budget requirements, interference limits, control/signalling channels, reserved handoff (guard) channels, redundancy, traffic growth, and backhaul and site-acquisition constraints.

### Trade-offs

- Blocking versus cost: the last 8 channels (92 → 100) cut blocking from 1.92 % to 0.40 % but raise provisioned capacity by 8.7 % and lower channel utilisation from 85 % to 80 %. Extra channels have strongly diminishing returns, which is the classic grade-of-service versus infrastructure trade-off.

- Trunking efficiency: large pooled groups are more efficient. Splitting 80 E into 3 equal sectors (26.7 E each, 2 % target) needs 36 channels each = 108 total; 4 cells of 20 E need 28 each = 112; 7 groups of 11.4 E need 18 each = 126, versus 92 for a single pool. Sectoring and cell splitting therefore consume more channels in total, and the expansion measure chosen in Stage 2 must be paid for in this way.

- Infrastructure: one 100-channel base station is enough for 80 E today; it can carry up to about 88 E at 2 % blocking (a 10 % growth margin). Growth beyond that requires a second group, sector or small cell.

Assumptions of the offered-traffic model: call arrivals are Poisson (random, independent) and holding times exponential; blocked calls are cleared with no retries; the user population is large relative to the channels (infinite-source approximation); every subscriber contributes the same 0.08 E (≈ 4.8 minutes of talk per busy hour) at the same time; users are spatially uniform and all served by this single group of channels; handoff traffic is ignored.

Stage 4 – Evaluate mobility and handoff	15 marks · K4/K5

QUESTION

Calculate the distance travelled during the 6 s overlap interval. Estimate hard handoffs per minute using the 1 km route cell length and supplied speed, stating how you interpret that length. Compare hard, soft and softer handoff in terms of connection continuity and resource use. Using the given interruption duration and 1 % target-cell unavailability, estimate a simple call-drop risk for one handoff and explain the limits of that estimate.

ANSWER

### Calculations

Speed: v = 60 km/h = 60 / 3.6 = 16.67 m/s.

Distance in the overlap interval: d = v·t = 16.67 m/s × 6 s = 100 m (about 10 % of a 1 km cell length). Interpretation of the 1 km length. I interpret 1 km as the nominal distance the mobile travels along the route between successive handoff points, i.e. one cell crossing per 1 km.

Handoff rate: v / L = (60 km/h) / (1 km) = 60 handoffs per hour = 1 hard handoff per minute (cell dwell time = 1000 m / 16.67 m/s = 60 s).

If 1 km were a cell diameter along a straight diametral path the answer would be the same; if the route cut across a smaller chord the rate would rise. The estimate also changes in proportion to speed and inversely with cell length – a 200 m microcell would give about 5 handoffs per minute at the same speed, which supports using umbrella cells for fast users.

### Hard, soft and softer handoff

| Type | How it works | Connection continuity | Resource use |
| --- | --- | --- | --- |
| Hard | Break-before-make: old link released, then new link set up (one radio link at a time; FDMA/TDMA, also used in LTE/NR with fast switching) | Short interruption (0.5 s in the data); failure if the target cell is unavailable | Lowest: only one channel per user; simple |
| Soft | Make-before-break: mobile connected to two or more cells (base stations) simultaneously (CDMA, same frequency) | Seamless; much lower drop risk | Higher: extra channels, backhaul and base-station processing in the overlap zone |
| Softer | Soft handoff between sectors of the same base station, combined at the base station | Seamless | Extra channel in sector but combining is local; no extra backhaul |

### Simple call-drop risk

Model: a handoff attempt succeeds if the target cell has a channel. With the target unavailable 1 % of the time, Pdrop ≈ 0.01 (1 %) per handoff for a single attempt. The 0.5 s interruption is short relative to the 6 s overlap window, so it is a brief gap or glitch in the speech stream rather than a drop on its own (it consumes 0.5/60 = 0.83 % of the cell dwell time). For a call of 10 minutes (about 10 handoffs at one per minute), the probability of at least one drop is 1 − 0.9910 ≈ 9.6 %; for 5 minutes, about 4.9 %.

Limits of the estimate. It assumes (i) a single attempt with no retries, whereas a mobile can retry during the 6 s overlap (which would reduce the risk if unavailability is short-lived, but not if the cell stays busy); (ii) independence between handoffs, although congestion is correlated in space and time; (iii) a constant 1 % unavailability that ignores traffic load; (iv) constant speed and a straight route with a clean overlap; and (v) no signal fading, ping-pong handoffs, hysteresis/threshold settings, or handoff-priority guard channels, all of which alter the real failure rate. The 1 % is also a single input, not a measurement. It is best used to compare options (for example whether to reserve guard channels or use soft handoff), not as a predicted drop rate.

## Stage 5 – Quantify interference and compare spectrum bands

#### QUESTION

Part A. For the interference case, calculate the carrier-to-interference ratio using the co-channel interferer alone. Then sum co-channel and adjacent-channel interference in linear power units, convert the total back to dBm, calculate SINR, and decide whether the 15 dB target is met. Recommend one suitable mitigation and explain its likely trade-off.

ANSWER
Part A – Interference calculation

- C/I with co-channel interference only: C/I = S − I₁ = −70 dBm − (−85 dBm) = 15 dB – exactly at the target, with zero margin.

mW

Convert to linear power (P	= 10dBm/10):

| Quantity | dBm | Linear (mW) |
| --- | --- | --- |
| Desired signal S | −70 | 1.000 × 10−7 |
| Co-channel I₁ | −85 | 3.162 × 10−9 |
| Adjacent-channel I₂ | −90 | 1.000 × 10−9 |
| Total interference Itot = I₁ + I₂ | −83.81 | 4.162 × 10−9 |

Itot(dBm) = 10 log₁₀(4.162 × 10−9) = −83.81 dBm

- SINR = S − Itot = −70 − (−83.81) = 13.81 dB (≈ 24 in linear terms).

Decision: 13.81 dB < 15 dB → the target is not met, a shortfall of 1.19 dB. The adjacent-channel term,

which appears small on its own, is enough to turn a just-compliant link into a failing one. Because thermal noise is not given and is excluded, the true SINR would be slightly lower still; the conclusion would only get worse.

Mitigation recommended: 120° sectorisation with antenna down-tilt (directional antennas that reduce received interference from other cells). To reach 15 dB the total interference must fall to −85 dBm (3.16 × 10−9 mW), i.e. a reduction of only 1.19 dB overall, or about 1.65 dB of co-channel interference if the adjacent-channel term stays fixed – sectorisation typically achieves this and more (reducing first-tier interferers from 6 to about 2 and by shaping patterns). Trade-off: channels are divided between sectors (lower trunking efficiency, see Stage 3), the site needs more antennas, radios and planning effort, and there are more intra-site handoffs (Stage 4). Alternatives are a larger reuse cluster (reduces capacity per cell), power control (reduces interference but also coverage) and better adjacent-channel filtering/guard bands (spends spectrum).

QUESTION

Part B. For 900 MHz, 3.5 GHz and 28 GHz, calculate wavelength and free-space path loss at 100 m, including path-loss differences relative to 900 MHz. Compare likely penetration, antenna size and coverage/capacity implications. Explain why the higher path loss at higher frequencies does not by itself determine achievable network performance.

ANSWER

### Part B – Spectrum-band comparison

λ = c/f ; FSPL(dB) = 20 log₁₀(4πd/λ), d = 100 m ; Δ relative to 900 MHz = 20 log₁₀(f/900 MHz).

| Band | Wavelength λ | FSPL at 100 m | Δ vs 900 MHz | Half-wave element (λ/2) |
| --- | --- | --- | --- | --- |
| 900 MHz | 0.333 m (33.3 cm) | 71.53 dB | 0 dB | ≈ 16.7 cm |
| 3.5 GHz | 0.0857 m (8.57 cm) | 83.32 dB | +11.80 dB | ≈ 4.3 cm |
| 28 GHz | 0.0107 m (1.07 cm) | 101.38 dB | +29.86 dB | ≈ 0.54 cm |

| Aspect | 900 MHz | 3.5 GHz | 28 GHz |
| --- | --- | --- | --- |
| Penetration | Best: passes walls and foliage with least loss | Moderate: adequate for outdoor-to-indoor with some loss | Poor: blocked by walls, glass, foliage, even the human body; mostly line-of-sight |
| Antenna size | Large antennas, few elements per panel | Compact; massive MIMO panels (64+ elements) practical | Tiny elements; very large arrays in small area, highly directive beams |
| Coverage | Wide area; largest cell, best for coverage and mobility | Medium cells (illustrative: radius about 0.5× that of 900 MHz for exponent 3.5–4 at equal power/gain); capacity layer | Small cells (hundreds of metres); hotspot/outdoor and fixed wireless |
| Capacity | Limited bandwidth (a few–tens of MHz) | Wide channels (e.g. tens to ≈100 MHz) → high capacity | Very wide channels (hundreds of MHz) → highest peak capacity but small coverage |

Why higher path loss does not alone determine performance. FSPL as defined assumes isotropic antennas of fixed gain. If the physical antenna aperture is held fixed, the Friis relation Pr = Pt·At·Ar/(λ²d²)

shows that received power rises with frequency, because shorter wavelengths allow far more array

elements, i.e. more beamforming gain, in the same area; this largely offsets the extra loss, for line-of-sight links at least. Achievable throughput also follows C = B·log₂(1+SINR), and higher bands offer much more bandwidth B; narrow beams reduce interference and improve spatial reuse; and the real path loss depends on blockage, penetration, rain/atmospheric absorption, and the path-loss exponent, not just frequency. Offsetting these, noise power rises with bandwidth and blockage limits mmWave coverage. Network performance is therefore decided by the full link budget, bandwidth, interference, load, beamforming and site density – not by FSPL alone.

## Stage 6 – Spectrum coordination and deployment recommendation

20 marks (with Stage 5 Part B) ·

K5/K6

QUESTION

Describe the distinct roles of ITU-R, the World Radiocommunication Conference (WRC) and the Radiocommunication Bureau (BR) in international spectrum coordination. Explain one way international coordination affects national regulators and operators. Conclude with a recommended staged strategy for the regional network. Your recommendation must connect generational evolution, the calculated cell and traffic plan, handoff behavior, interference results, and spectrum-band characteristics. Identify the most important assumptions or uncertainties that should be checked before deployment.

ANSWER

### International spectrum coordination

| Body | Distinct role |
| --- | --- |
| ITU-R (Radiocommunication Sector of the ITU) | Standing sector of the ITU (a UN specialised agency). Through its study groups it produces technical studies, Recommendations and Reports (e.g. the IMT-2020 and IMT-2030 frameworks) and maintains the technical basis for the Radio Regulations. |
| WRC (World Radiocommunication Conference) | Treaty-making conference of Member States, normally held every three to four years. It reviews and revises the Radio Regulations: the international Table of Frequency Allocations, identification of bands for services such as IMT, and technical/regulatory conditions. |
| BR (Radiocommunication Bureau) | The ITU-R’s executive/administrative arm under an elected Director. It applies the Radio Regulations: processes frequency-assignment and satellite-network notifications, records them in the Master International Frequency Register, supports coordination between administrations, assists in resolving harmful-interference cases, and prepares and supports study groups and WRCs. |

How coordination affects regulators and operators (one example). When a WRC identifies a band for IMT (for example 24.25–27.5 GHz was identified at WRC-19, and the 3.3–3.8 GHz range is identified in many regions), national regulators typically follow the allocation and conditions in their own licensing and auctions, and must coordinate border use and protection of other services (e.g. satellite earth stations near 3.5 GHz). For the operator this means harmonised bands, a larger device ecosystem and lower equipment cost, but also licence conditions, power/emission limits and cross-border coordination obligations. The exact status of 28 GHz, in particular, differs by country and should be checked in the national frequency table.

### Recommended staged deployment strategy

| Phase | Action | Evidence from this report |
| --- | --- | --- |
| 0. Verify (before build) | Measure real busy-hour traffic, propagation exponent, interference and route handoffs; confirm spectrum licences and ITU/national band status | Results below depend on assumed 0.08 E/user, ideal hexagon and one interference snapshot |
| 1. Coverage foundation | 900 MHz LTE/NR macro layer on hexagonal sites, r ≈ 2 km (10.39 km² per site), 3-sector antennas with down-tilt; one 100-channel-equivalent group per site; reuse N = 7 (D = 9.17 km) | Lowest FSPL (71.5 dB) and best penetration; 80 E needs 92 channels (1 group, 0.40 % blocking at 100); 1 handoff/min at 60 km/h is manageable |
| 2. Capacity layer | 3.5 GHz NR with massive-MIMO beamforming co-located on the same macro sites | +11.8 dB loss versus 900 MHz offset by array gain; wider channels give the main capacity gain |
| 3. Hotspot densification | 28 GHz small cells/microcells/picocells only at dense or fixed-wireless locations and indoor venues; umbrella macro for mobility; femtocells only if coordinated | +29.9 dB loss and blockage limit range; at 60 km/h a small cell would cause several handoffs per minute |
| 4. 6G readiness | Keep fibre backhaul, energy efficiency and site flexibility; add AI-driven optimisation and ISAC/THz trials as standards (IMT-2030) mature | 6G is a vision, not yet a standard |

Integration of results. (1) Evolution: the layered low-/mid-/mmWave plan follows 1G–5G history: capacity from more spectrum, better efficiency and more spatial reuse. (2) Cell and traffic: D = 9.17 km at N = 7; 80 E needs 92 channels, so one 100-channel group suffices today (about 108 channels with 3 sectors). (3) Handoff: 1 handoff/min at about 1 % failure gives about 9.6 % drop chance in a 10-minute call, so use guard channels and handoff priority. (4) Interference: baseline SINR 13.81 dB fails 15 dB; sectorisation/down-tilt (about 1.2–1.7 dB needed) fixes it at the cost of trunking efficiency, and a margin beyond 15 dB should be designed in. (5) Spectrum: 900 MHz for coverage, 3.5 GHz for capacity, 28 GHz for hotspots, subject to ITU/national licensing.

#### Most important assumptions and uncertainties to check before deployment:

- Traffic: 0.08 E per user, 1,000 users and Poisson/exponential statistics; true busy-hour peaks, hotspots and growth will change the channel and site count.

- Propagation and coverage: the ideal hexagon, free-space path loss and flat terrain; real clutter, shadowing and penetration loss need drive tests or a planning tool; guaranteed coverage must be shown by a link budget.

- Interference: the −85 dBm and −90 dBm values are one snapshot; thermal noise and aggregate interference from all six first-tier cells are not included; margin should be verified in the field.

- Mobility: the 1 % target-unavailability and 0.5 s interruption are single inputs; actual handoff failure depends on load, thresholds, hysteresis and route geometry.

- Spectrum and regulation: availability, licence terms, power limits and cross-border/satellite coordination of 900 MHz, 3.5 GHz and 28 GHz in the operator’s country; device ecosystem support.

- Economics: site, backhaul and energy cost (not analysed here), particularly for 28 GHz densification.

### References

T. S. Rappaport, Wireless Communications: Principles and Practice, 2nd ed., Prentice Hall (hexagonal geometry, reuse, Erlang B, handoff, path loss).

ITU-R Radio Regulations (edition in force) and ITU-R Recommendations M.2083 (IMT-2020 vision) and M.2160 (IMT-2030 framework) – general descriptions of ITU-R, WRC and BR roles and IMT work.

3GPP specifications for LTE and NR (e.g. TS 38.300) – general overview of OFDMA, MIMO, beamforming, eMBB/URLLC/mMTC.

These are standard textbook/standards-body references used from general knowledge; band-status and licensing details must be checked against current ITU and national sources.